END SEMESTER ASSESSMENT (ESA) B.TECH. (CSE)

IV SEMESTER

UE18CS256 – MICROPROCESSOR AND COMPUTER ARCHITECTURE LABORATORY

PROJECT REPORT

ON

BUILDING SECURITY SYSTEM USING ARDUINO

SUBMITTED BY

NAME 	     SRN

- Student1  Achyut Jagini                    PES2UG19CS013

- Student2  Amar S Sajjanshetty       PES2UG19CS032

- Student3

- Student4

JANUARY – MAY 2021

DEPARTMENT OF COMPUTER SCIENCE & ENGINEERING

ELECTRONIC CITY CAMPUS,

BENGALURU – 560100, KARNATAKA, INDIA

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
| Sl.No | TOPIC | PAGE No |
|  | ABSTRACT OF THE PROJECT |  |
|  | CIRCUIT DIAGRAM |  |
|  | ARDUINO CODE |  |
|  | SCREEN SHOTS OF THE OUTPUT |  |
|  | REFERENCES |  |

ABSTRACT OF THE PROJECT:

# This project is implemented using Arduino circuit board.

# In this project we made “ BUILDING SECURITY SYSTEM ” using following sensors.

1) Arduino boards

2)LCD screen

3)Buzzer

4)Servo motors.

5) Number Keypad

6)Gas sensor

7)Led lights

8)Bread boards.

# Our project helps to maintain security of the building.

#We have used 2 Arduino boards to implement the security

system. First arduino board is connected with number keypad, servo motor (Main door), buzzer. The second Arduino board is connected with leds (indicator), pair of servo motors (emergency exit doors), gas sensors etc

# We have installed keypad with LCD screen to prompt the user to type his password to enter into the building.

If the user fails to enter the correct password multiple times,

the system will lock and the door remains closed.

# If any intruder tries to enter into the building and types incorrect password multiple times, then the system buzzer starts to make a emergency sound.

#  If the building catches fire accidently or by terrorists (manually) the gas sensor  will sense the smoke from the building and this will automatically opens the exit door,

so that the people who were residing inside the building can escape out from the building and save there lives.

CIRCUIT DIAGRAM:

ARDUINO CODE:

#include <LiquidCrystal.h>

#include <Keypad.h>

#include <Servo.h>

// initialize the library with the numbers of the interface pins

LiquidCrystal LCD( 8, 9, 10, 11, 12, 13 ) ;

Servo servo ;

const int buzzerPin = 14 ;

const int servoPin = 15 ;

const byte ROWS = 4, COLS = 4 ;

// Define the Keymap

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

} ;

byte rowPins[ROWS] = { 16, 17, 2, 3 } ;

byte colPins[COLS] = { 4, 5, 6, 7 } ;

//  Create the Keypad

Keypad kpd = Keypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS ) ;

char password[16], string[16] ;

int flag_h_setpassword = 1, flag_inputpassword = 0, flag_inputstring = 0, flag_opendoor = 1, flag_state = 0, flag_remoteopen = 0, flag_lockdown = 0 ;

int count = 0, trial_count=0, pos = 0, state = 0 ;

// flag_setpassword = flag for setting the password,

// flag_stringinput = flag for taking input the string,

void setup() {

for(int k=8 ; k<14 ; k++) {

pinMode(k,OUTPUT) ;   //pins 8-14 are enabled as output

}

LCD.begin(16, 2) ;   //initializing LCD

// for the Peizo-buzzer

//Serial.begin(8600) ;

pinMode(buzzerPin, OUTPUT) ;

// for Servo-Motor

//pinMode(servoPin, OUTPUT) ;

servo.attach(servoPin) ;

// Now, the Welcome message...

LCD.setCursor(0,0) ;

LCD.print("   WELCOME !!") ;

LCD.setCursor(0,1) ;

LCD.print("Set a Password :") ;

InitializePassword(), InitializeString() ;

CloseDoor() ;

}

void loop() {

if( Serial.available() > 0 ) {

state = Serial.read() ;

flag_state = 0 ;

}

//if( state != 0 )   flag_lockdown = 1 ;

// ##### Keypad #####

if( trial_count < 3 ) {

char key = kpd.getKey() ;   //storing pressed key value in a char

if( key != NO_KEY ) {

if( flag_h_setpassword == 1 ) {

H_SetPassword() ;

}

if( key == '*' ) {

if( flag_inputpassword == 1 ) {

InitializePassword(), H_SetPassword() ;

}

else if( flag_inputstring = 1 ) {

InitializeString(), H_EnterPassword() ;

}

}

else if( key == '#' ) {

if( flag_inputpassword == 1  &&  count > 0 ) {

flag_inputpassword = 0 ;

password[count] = '\0' ;

H_EnterPassword() ;

}

else if( flag_inputstring == 1  &&  count > 0 ) {

flag_inputstring = 0 ;

string[count] = '\0' ;

if( Compare_Password_and_String() == 1 ) {

LCD.clear() ;

LCD.print("***VERIFIED!!***") ;

Serial.println("UNLOCKED !!") ;

trial_count = 0 ;

for( int i=0 ; i<3 ; ++i ) {

tone(buzzerPin, 500, 200) ;

delay(230) ;

tone(buzzerPin, 100, 200) ;

delay(300) ;

OpenDoor() ;

}

}

else {

LCD.clear() ;

LCD.print("Wrong Password !") ;

Serial.println("Someone unsuccessfully attempted to open the lock !") ;

++trial_count ;

tone(buzzerPin, 100, 1000) ;

delay(1000) ;

H_EnterPassword() ;

}

}

}

else if( flag_inputpassword == 1  ||  flag_inputstring == 1 ) {

LCD.print(key) ;

delay(100) ;

LCD.setCursor(count,1) ;

LCD.print('*') ;

if( flag_inputpassword == 1 )   password[count] = key ;

else if( flag_inputstring == 1 )   string[count] = key ;

++count ;

}

}

}

else {

LCD.clear() ;

LCD.setCursor(0,0) ;

LCD.print("SYSTEM LOCKDOWN!") ;

tone(buzzerPin, 1000, 1000) ;

delay(1500) ;

flag_lockdown = 1 ;

}

}

void InitializePassword() {

for( int i=0 ; i<16 ; ++i )

password[i] = 0 ;

}

void InitializeString() {

for( int i=0 ; i<16 ; ++i )

string[i] = 1 ;

}

void H_SetPassword() {

LCD.clear() ;

LCD.setCursor(0,0) ;

LCD.print("Set a Password :") ;

LCD.setCursor(0,1) ;

flag_h_setpassword = 0 ;

flag_inputpassword = 1, count = 0 ;

}

void H_EnterPassword() {

CloseDoor() ;

LCD.clear() ;

LCD.setCursor(0,0) ;

LCD.print("Enter Password :") ;

LCD.setCursor(0,1) ;

flag_inputstring = 1, count = 0 ;

}

int Compare_Password_and_String() {

int i ;

for( i=0  ;  password[i]!='\0' && string[i]!='\0'  ;  ++i ) {

if( password[i] != string[i] )

return 0 ;

}

if( password[i] == '\0'  &&  string[i] == '\0' )

return 1 ;

else   return 0 ;

}

void OpenDoor() {

if( flag_opendoor == 1 )

return;

for( pos=15 ; pos<=100 ; ++pos ) {

servo.write(pos) ;

delay(15) ;

}

flag_opendoor = 1 ;

}

void CloseDoor()  {

if( flag_opendoor == 0 )

return;

for( pos=100 ; pos>=15; --pos ) {

servo.write(pos) ;

delay(15) ;

}

flag_opendoor = 0 ;

}

2 Aurdino

#include<Servo.h>

int sensor=0;

Servo m1;

Servo m2;

int pos1 = 0;

int pos2 = 0;

int flago1=0;

int flago2=0;

void setup() {

m1.attach(9);

m2.attach(10);

pinMode(13, OUTPUT);

Serial.begin(9600);

}

void loop() {

sensor = analogRead(A0);

if (sensor > 250)

{

digitalWrite(13, HIGH);

Serial.println(sensor);

open1();

Serial.print("FIRE !! Please Exit ");

}

else {

digitalWrite(13, LOW);

close1();

}

}

void open1()

{

m1.write(90);

m2.write(90);

}

void close1()

{

m1.write(0);

m2.write(0);

}

SCREEN SHOTS OF THE OUTPUT:

welcome.set password

Set password

enter password

Verified as correct password

Wrong password

System lockdown if wrong password entered 3 times

REFERENCES

https://en.wikipedia.org/wiki/Arduino

https://www.arduino.cc/en/guide/introduction

https://learn.sparkfun.com/tutorials/what-is-an-arduino/all

https://www.seeedstudio.com/blog/2019/12/04/introduction-to-the-arduino-what-is-arduino/